CISCO MODELING LABS  ·  BEHAVIORAL VALIDATION

Stretched Firewall HA

Dual-site Active/Standby failover — lab validation results

2 FTD pairs across East & West    ·    FTDv 7.7 / FMC 10.0.1    ·    25 ms interconnect    ·    TRex + iperf3 under load    ·    2026-09-09

# Summary

Cross-site stateful failover holds at 25 ms. A planned failover moves the active unit between sites in about 5 seconds and established sessions survive, with no measurable degradation from 3 ms to 40 ms of round-trip latency. Latency is not the risk. The real exposures are the shared interconnect, where a full cut puts both pairs into split-brain, and edge-router failure, which blackholes a site until routing detects it. Both have concrete mitigations, given in the findings.

| ~5 s Planned failover — active moves site | 0 Established sessions dropped (state link up) |
| --- | --- |
| 3–40 ms Failover time flat across this range | ~16 s Router-failure outage after mitigation (from minutes) |

# The design being validated

The internet edge is built as two FTD Active/Standby pairs, each split across the two colocation sites. Pair A runs active in East with its standby in West and fronts the East-homed services; pair B is the mirror image, active in West. Both sites carry live traffic in steady state because each hosts one pair's active unit. the enterprise's public space is divided into a pair A portion and a pair B portion; each portion is advertised from both sites, unprepended at its home site and AS-path prepended at the other, so each ISP delivers a portion to its home site. The colocation layer 2 interconnect carries every stretched VLAN.

Figure 1. Two sites, two split failover pairs, per-pair ingress preference, colocation L2 interconnect.

Per site, two Catalyst 8500 routers (one per ISP, eBGP up and iBGP between them) feed an outside layer 2 domain carrying both pairs' outside VLANs, one unit from each pair, and an inside layer 2 domain carrying both pairs' inside VLANs plus all four HA VLANs. Both domains extend to the peer site over the interconnect, which in production is a single 802.1Q trunk circuit. Each pair owns four VLANs — outside, inside, failover, and state — and the pairs share nothing: no VLAN, subnet, public block, or gateway. Failover and state are kept on separate links so state replication can be lost without losing the failover heartbeat.

Figure 2. Per-site connectivity (East shown; West is the mirror image).

Why it needs validating. Cross-site Active/Standby was selected over clustering, which the 20-to-25 ms round trip rules out, and over site-local models that cannot preserve sessions across a site loss. Its known cost is concentration of risk on the inter-site circuit: it carries both pairs' data VLANs, both pairs' failover and state links, HSRP hellos, and spanning tree. The lab was built to reproduce that shared-fate topology at a measured 25 ms and observe how failover, session preservation, split-brain, and inbound path movement actually behave, rather than argue them on paper.

# Test results

Nine scenarios, each run with live HTTP load and a long-lived session through pair A. “Session” shows whether that held connection kept its state across the event or reset.

| Test | Scenario | Result | Detect | Session | What it means |
| --- | --- | --- | --- | --- | --- |
| T1 | Baseline health | stable | — | held | Both pairs Active/Standby, state sync current, no flapping. |
| T2 | Planned failover | pass | 5.2 s | survived | Active role moved East→West; brief throughput dip as Snort rebuilt inspection. |
| T3 | Failover, state link down | by design | 4.8 s | reset | Without state sync the session dropped — the value of the state link, shown directly. |
| T4 | Failover link only cut | pass | none | held | No split-brain: hellos also cross the data interfaces, so the standby stayed standby. |
| T5 | Full interconnect cut | risk shown | 16.8 s | held* | Split-brain on both pairs — the realistic single-circuit failure. *This flow's path never crossed the cut. |
| T6 | Site failure (East off) | pass | 1.2 s | survived | Pair A recovered to West; ~30–40 s to full traffic. Pair B untouched. |
| T7 | Latency sweep 3–40 ms | pass | ~4.7 s | survived | Failover time and survival unchanged across the range. Latency is not the risk. |
| T8 | Degraded link 1% / 5% loss | pass | none | held | No false failover at either loss rate; detection tolerates a lossy circuit. |
| T9 | Edge-router failure | mitigated | — | reset→held | Default timers blackholed the site; fast BGP detection cut it to ~16 s (see below). |

Behavioral results on virtual platforms (FTDv in Cisco Modeling Labs); demonstrate behavior, not hardware FTD performance or capacity. No figure here is sizing evidence.

# The two results that decide the design

Failover versus latency. Planned failover was repeated at five round-trip times — roughly 3, 6, 10, 25 and 40 ms. Detection stayed near 4.7 seconds and every session survived. The line is flat, which is the finding: within this range latency does not degrade planned stateful failover, because the state link that crosses the WAN syncs well inside Cisco's 250 ms ceiling.

Edge-router failure. Powering off one edge router, measured by routing-failure-detection setting:

| Routing failure detection | Inbound outage | Connections lost |  |
| --- | --- | --- | --- |
| Default timers, no BFD | no recovery | 4,475 |  |
| BFD on eBGP only | ~100 s | 2,653 |  |
| BGP timers 3/9 everywhere | ~16 s | 193 | recommended |

Firewall HA does not cover the routers. With default BGP timers the site blackholed for minutes because routing never detected the dead router; the firewalls were healthy the whole time. Fast detection is the fix.

# Findings & recommendations

Confirmed  Latency is not the problem

Planned failover preserved sessions at every latency to 40 ms. 25 ms is a sound design point for cross-site Active/Standby.

Risk  One circuit means split-brain

A full interconnect cut drove both pairs active at once. Everything rides one inter-site path today. Procure diverse physical circuits and place the failover and state VLANs on the second path.

Mitigated  Pair firewall HA with fast routing detection

An abrupt edge-router loss blackholed the site on default timers. Aggressive BGP timers (and BFD where the platform honors fall-over) cut the outage to about 16 seconds.

Design note  Keep failover and state on separate links

With the state link cut, the next failover reset every session; with it up, sessions survived. Separate links let one fail without the other.

# Appendix A — VLAN plan

Eight VLANs, four per pair. All eight are stretched across both sites over the colocation layer 2 interconnect.

| VLAN | Name | Pair | Purpose |
| --- | --- | --- | --- |
| 100 | OUTSIDE-A | A | Pair A outside / public; HSRP group 10 gateway (active in East) |
| 110 | INSIDE-A | A | Pair A inside / private |
| 900 | FAILOVER-A | A | Pair A failover heartbeat link |
| 901 | STATE-A | A | Pair A stateful failover (session replication) |
| 200 | OUTSIDE-B | B | Pair B outside / public; HSRP group 20 gateway (active in West) |
| 210 | INSIDE-B | B | Pair B inside / private |
| 902 | FAILOVER-B | B | Pair B failover heartbeat link |
| 903 | STATE-B | B | Pair B stateful failover (session replication) |

# Appendix B — Addressing plan

Ranges shown are the lab's documentation stand-ins (RFC 5737 public, RFC 1918 private) for the enterprise's real address space. The production prefix split between the two pair portions is a low-level design item.

| Segment | Range (lab) | Notes |
| --- | --- | --- |
| Pair A public (VLAN 100) | 203.0.113.0/26 | enterprise public space, pair A portion |
| Pair B public (VLAN 200) | 203.0.113.64/26 | enterprise public space, pair B portion |
| Pair A inside (VLAN 110) | 10.10.0.0/24 | Private, behind pair A NAT |
| Pair B inside (VLAN 210) | 10.20.0.0/24 | Private, behind pair B NAT |
| Pair A failover / state | 192.168.90.0/30, 192.168.91.0/30 | HA links (900 / 901) |
| Pair B failover / state | 192.168.92.0/30, 192.168.93.0/30 | HA links (902 / 903) |
| ISP transit links | 198.51.100.0/24 | eBGP to both providers |
| Simulated internet clients | 192.0.2.0/24 | Lab traffic sources |